NC Division of Energy, Mineral, and Land Resources

Operation & Maintenance Inspection Form – Bioretention Cell

| Project name: | Permit number: SW | Inspection date: |
| --- | --- | --- |
| Street address: | City: | State and ZIP code: |
| Name of inspector: | Phone number: | Email address: |
| Description of SCM location on the project: |  |  |
| Signature of Inspector certifying that the information provided in this form is complete and correct: |  |  |

| Stormwater Collection System, Accessibility, and Overall Cell |  |  |  |
| --- | --- | --- | --- |
| - Swales are free from blockage and erosion. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Grates, gutters, curb openings, and pipes appear free from damage & clogs. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Bioretention cell is accessible for inspection. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Bioretention cell is free from trash and debris. |  |  | Yes ☐ No ☐ N/A ☐ |
| Inlet(s) and Pretreatment |  |  |  |
| - Inlet(s) are free from damage, clogs, and erosion. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Flow is directed to a pretreatment area (if applicable). |  |  | Yes ☐ No ☐ N/A ☐ |
| - Pretreatment area is free from erosion and clogging (if applicable). |  |  | Yes ☐ No ☐ N/A ☐ |
| - Pretreatment area has < 2 inches of accumulated sediment (if applicable). |  |  | Yes ☐ No ☐ N/A ☐ |
| Bioretention Cell if Vegetation is Grass |  |  |  |
| - Grass is healthy. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Grass is not fertilized. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Base of grass is at least 10 inches below the outlet structure. |  |  | Yes ☐ No ☐ N/A ☐ |
| Bioretention Cell if Vegetation is Trees & Shrubs |  |  |  |
| - A two- to-four-inch layer of mulch is present. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Mulch is not clogged with sediment. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Cell is free of weeds and obligate wetland vegetation. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Vegetation is healthy and not overgrown, especially if safety is an issue. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Tree stakes/wires are not present (remove after 6 months). |  |  | Yes ☐ No ☐ N/A ☐ |
| - Vegetation is not fertilized. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Top of the mulch is at least 10 inches below the outlet structure. |  |  | Yes ☐ No ☐ N/A ☐ |
| Underdrain and Outlet |  |  |  |
| - Standing water is not present in the bioretention cell. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Underdrains appear free from damage and clogs. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Clean out/ observation pipes are not damaged. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Emergency bypass and/or bypass swale is clearly visible & in good condition. |  |  | Yes ☐ No ☐ N/A ☐ |
| - Outlet structure is free from damage, clogs, and erosion. |  |  | Yes ☐ No ☐ N/A ☐ |
| Action Plan to address any “Nos” listed above, including dates for completion (for permittee to complete): |  |  |