NC Division of Energy, Mineral, and Land Resources

Operation & Maintenance Inspection Form – Wet Pond

| Project name: |  | Permit number: SW | Inspection date: |
| --- | --- | --- | --- |
| Street address: |  | City: | State and ZIP code: |
| Name of inspector: |  | Phone number: | Email address: |
| Required forebay depth: | Measured forebay depth: | Required main pool depth: | Measured main pool depth: |
| Description of SCM location on the project: |  |  |  |
| Signature of Inspector certifying that the information provided in this form is complete and correct: |  |  |  |

| Stormwater Collection System and Accessibility |  |  |
| --- | --- | --- |
| - Swales are free from blockage and erosion. |  | Yes ☐ No ☐ N/A ☐ |
| - Grates, gutters, curb openings, and pipes appear free from damage and clogs. |  | Yes ☐ No ☐ N/A ☐ |
| - Wet pond is accessible for inspection. |  | Yes ☐ No ☐ N/A ☐ |
| Inlet(s) and Forebay |  |  |
| - Inlet(s) are free from damage, clogs, and erosion. |  | Yes ☐ No ☐ N/A ☐ |
| - Forebay berm is stable and not eroding. |  | Yes ☐ No ☐ N/A ☐ |
| - Algal growth covers < 50% of the surface area. |  | Yes ☐ No ☐ N/A ☐ |
| - Cattails, phragmites, or other invasive plants cover < 10% of the surface area. |  | Yes ☐ No ☐ N/A ☐ |
| - Forebay is free from trash and debris. |  | Yes ☐ No ☐ N/A ☐ |
| - Forebay is at least 2 feet deep. |  | Yes ☐ No ☐ N/A ☐ |
| Perimeter, Embankment, and Side Slopes |  |  |
| - Perimeter is free from water quality impacts caused by geese and ducks. |  | Yes ☐ No ☐ N/A ☐ |
| - Embankment is free from woody shrubs and trees. |  | Yes ☐ No ☐ N/A ☐ |
| - Side slopes are stable, vegetated, and free from erosion/bare soil. |  | Yes ☐ No ☐ N/A ☐ |
| - Side slopes are free from trash and debris. |  | Yes ☐ No ☐ N/A ☐ |
| - Side slopes are free from muskrat and/or beaver activity. |  | Yes ☐ No ☐ N/A ☐ |
| Main Pool |  |  |
| - Vegetated shelf is covered with healthy vegetation and free from erosion. |  | Yes ☐ No ☐ N/A ☐ |
| - Algal growth covers < 20% of the surface area. |  | Yes ☐ No ☐ N/A ☐ |
| - Cattails, phragmites, or other invasive plants cover < 10% of the surface area. |  | Yes ☐ No ☐ N/A ☐ |
| - Main pool is free from trash and debris. |  | Yes ☐ No ☐ N/A ☐ |
| - Measured main pool depth > required main pool depth. |  | Yes ☐ No ☐ N/A ☐ |
| - Fountain is not causing sedimentation problems in the pond. |  | Yes ☐ No ☐ N/A ☐ |
| Outlet Device(s) |  |  |
| - Emergency bypass and/or bypass swale is clearly visible and in good condition. |  | Yes ☐ No ☐ N/A ☐ |
| - Outlet structure is free from damage, clogs, and erosion. |  | Yes ☐ No ☐ N/A ☐ |
| - Water level matches the level of the lowest outlet. |  | Yes ☐ No ☐ N/A ☐ |
| Action Plan to address any “Nos” listed above, including dates for completion (for permittee to complete): |  |