NC Division of Energy, Mineral, and Land Resources

Operation & Maintenance Inspection Form – Stormwater Wetland

| Project name: | Permit number: SW | Inspection date: |
| --- | --- | --- |
| Street address: | City: | State and ZIP code: |
| Name of inspector: | Phone number: | Email address: |
| Description of SCM location on the project: |  |  |
| Signature of Inspector certifying that the information provided in this form is complete and correct: |  |  |

| Stormwater Collection System and Accessibility |  |
| --- | --- |
| - Swales are free from blockage and erosion. | Yes ☐ No ☐ N/A ☐ |
| - Grates, gutters, curb openings, and pipes appear free from damage and clogs. | Yes ☐ No ☐ N/A ☐ |
| - The stormwater wetland is accessible for inspection. | Yes ☐ No ☐ N/A ☐ |
| Inlet(s) and Forebay |  |
| - Inlet(s) are free from damage, clogs, and erosion. | Yes ☐ No ☐ N/A ☐ |
| - Forebay berm is stable and not eroding. | Yes ☐ No ☐ N/A ☐ |
| - Cattails, phragmites or other invasive plants cover < 10% of the surface area. | Yes ☐ No ☐ N/A ☐ |
| - Forebay is free from trash and debris. | Yes ☐ No ☐ N/A ☐ |
| - Forebay is at least 15 inches deep. | Yes ☐ No ☐ N/A ☐ |
| Perimeter, Embankment, and Side Slopes |  |
| - Perimeter is free from water quality impacts caused by geese and ducks. | Yes ☐ No ☐ N/A ☐ |
| - Embankment is free from woody shrubs and trees. | Yes ☐ No ☐ N/A ☐ |
| - Side slopes are stable, vegetated, and free from erosion/bare soil. | Yes ☐ No ☐ N/A ☐ |
| - Side slopes are free from trash and debris. | Yes ☐ No ☐ N/A ☐ |
| - Side slopes are free from muskrat and/or beaver activity. | Yes ☐ No ☐ N/A ☐ |
| Shallow Water and Temporary Inundation Zone(s) |  |
| - Shallow water/temporary inundation zones are covered with healthy vegetation. | Yes ☐ No ☐ N/A ☐ |
| - Shallow water and temporary inundation zones are free from erosion. | Yes ☐ No ☐ N/A ☐ |
| - Cattails, phragmites or other invasive plants cover < 10% of the surface area. | Yes ☐ No ☐ N/A ☐ |
| - Temporary inundation zones are drained within 5 days after a storm event. | Yes ☐ No ☐ N/A ☐ |
| - Shallow water and temporary inundation zones are free from trash and debris. | Yes ☐ No ☐ N/A ☐ |
| Deep Pools |  |
| - Deep pools are at least 15 inches deep. | Yes ☐ No ☐ N/A ☐ |
| - Cattails, phragmites, or other invasive plants cover < 10% of the surface area. | Yes ☐ No ☐ N/A ☐ |
| - Deep pools are free from trash and debris. | Yes ☐ No ☐ N/A ☐ |
| - Standing water is present in deep pools. | Yes ☐ No ☐ N/A ☐ |
| - Deep pool adjacent to the outlet structure is free of vegetation. | Yes ☐ No ☐ N/A ☐ |

| Outlet Device(s) |  |  |  |
| --- | --- | --- | --- |
| - Emergency bypass and/or bypass swale is clearly visible and in good condition. |  | Yes ☐ No ☐ N/A ☐ |  |
| - Outlet structure is free from damage, clogs, and erosion. |  | Yes ☐ No ☐ N/A ☐ |  |
| - Water level matches the level of the lowest outlet. |  | Yes ☐ No ☐ N/A ☐ |  |
| Action Plan to address any “Nos” listed above, including dates for completion (for permittee to complete): |  |  |