Potential MDC for Green Roofs

for MDC Team discussion on January 12, 2015

- SLOPE.  The green roof shall have a slope (or pitch) of no greater than 15 percent.  A blue roof that is not flat has limited storage effectiveness.

- MEDIA SPECIFICATION.  Green roof media is an engineered media comprised of 80-95% (by volume) light-weight aggregate (LWA) and 5-10% (by volume) organic matter. The maximum organic fraction of the media shall be limited to 10% by volume.

- MINIMUM MEDIA DEPTH.  The minimum media depth for a new green roof shall be 4 inches.  If an existing roof is unable to structurally support at least 4 inches of media, then a shallower media depth is acceptable.

- VEGETATION SPECIFICATION.  A vegetation plan for the green roof shall be prepared by a licensed professional who is familiar with green roof vegetation and conditions. ASTM E2400-06 Standard Guide for Selection, Installation, and Maintenance of Plants for Green Roof Systems may be used as a reference. Vegetation may need irrigation during establishment and throughout much of the roof’s life.

- MAXIMUM WATER STORAGE.  The maximum water storage in the growing media per unit area of green roof shall not exceed 1.5 inches.

- PROTECTION OF ROOF DRAINAGE.  Roof drainage features such as inlets, gutters, pipes shall be protected from intrusion by vegetation roots and growing media.

- CONSTRUCTION PHASING.  Green roofs should be the final portion of the roof system installed to prevent excessive trampling of vegetation and compaction of media.

- SAFETY.  Barriers or other safety devices shall be installed to protect people from falling off the roof.

- While green roofs may be designed to receive some runoff from adjoining elevated roof surfaces.

- RECOMMENDATION.  A waterproof membrane should be installed between the building structure and the drainage layer of a green roof to protect the building structure from leaks