Potential MDC for Sand Filters

November 17, 2014

Blue font = from Bioretention MDC

- SHWT SEPARATION.  The SHWT shall be at least two feet below the bottom of the sand filter for open-bottom designs and one foot separation is required for closed-bottom designs. Exceptions to the one foot SHWT separation may be made if the sand filter does not drain the water table and it does not float.

- EXCESS VOLUME/FLOW ATTENTUATION.  Volume in excess of the treatment volume shall be bypassed.  OR borrow from bioretention MDC:  “Bioretention cells may store peak attenuation volume at a depth of up to 24 inches above the media surface.  The peak attenuation outlet shall be a maximum of 18 inches above the media surface.

- MAXIMUM DRAINAGE AREA SIZE.  The maximum contributing drainage area is five acres.  One acre or less is recommended.

- TWO CHAMBER SYSTEM.  The sand filter shall include a sedimentation chamber and a sand chamber.

- MAXIMUM PONDING DEPTH.  The maximum ponding depth for 75% of the treatment volume shall be six feet.  The ponding depth is typically measured from the top of the overflow weir that separates the sediment chamber from the sand chamber.

- DRAINAGE TIME.  The sand filter must completely drain within 40 hours.

- SEDIMENTATION CHAMBER.  The sedimentation chamber shall contain ponded water.

- SEDIMENT CHAMBER SIZING.  The size of the sediment chamber shall be:

treatment volume/(15 feet)

- MINIMUM WIDTH OF SEDIMENTATION CHAMBER.  The minimum width (parallel to flow) of the sedimentation chamber shall be 1.5 feet.

- FLOW DISTRIBUTION.  Incoming stormwater shall be evenly distributed over the surface of the sand chamber.

- SAND MEDIA SPECIFICATION.  Sand media shall be clean, washed, course masonry sand such as ASTM C33.  The phosphorus index (P-index) for the media shall be between 10 and 50 if the receiving water is class B, C, SB or SC with no supplementary classification.  Otherwise, the P-index shall be between 10 and 30.

- MEDIA DEPTH.  The filter bed shall have a minimum depth of 18 inches, with a minimum depth of sand above the drainage pipe of 12 inches.

- SAND CHAMBER SIZING.  The size of the sand chamber shall be:

(treatment volume) * (media depth)           .

140 feet * (maximum ponding depth + media depth)

- MAINTENANCE OF MEDIA.  The sand filter shall be maintained in a manner that results in a drawdown of at least one inch per hour at the planting surface.

- UNDERDRAIN.  An underdrain with internal water storage shall be installed unless it can be demonstrated that the in-situ soil infiltration rate is two inches per hour or greater immediately prior to the initial placement of the media. The internal water storage zone shall extend to a minimum of 18” below the planting surface.

- CLEAN-OUT PIPES.  At least one clean-out pipe shall be provided on each line.  Clean out pipes shall be capped.