| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Inorganics | Metals | Metals by ICP | Aluminum | EPA 200.7, 4.4 (1994) | 50 ug/L |  |
| Inorganics | Metals | Metals by ICP | Barium | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP | Beryllium | EPA 200.7, 4.4 (1994) | 5 ug/L |  |
| Inorganics | Metals | Metals by ICP | Boron | EPA 200.7, 4.4 (1994) | 50 ug/L |  |
| Inorganics | Metals | Metals by ICP | Calcium * | EPA 200.7, 4.4 (1994) | 0.1 mg/L |  |
| Inorganics | Metals | Metals by ICP | Chromium | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP | Cobalt | EPA 200.7, 4.4 (1994) | 50 ug/L |  |
| Inorganics | Metals | Metals by ICP | Copper | EPA 200.7, 4.4 (1994) | 20 ug/L |  |
| Inorganics | Metals | Metals by ICP | Iron | EPA 200.7, 4.4 (1994) | 50 ug/L |  |
| Inorganics | Metals | Metals by ICP | Lead | EPA 200.7, 4.4 (1994) | 25 ug/L |  |
| Inorganics | Metals | Metals by ICP | Lithium | EPA 200.7, 4.4 (1994) | 25 ug/L |  |
| Inorganics | Metals | Metals by ICP | Magnesium * | EPA 200.7, 4.4 (1994) | 0.1 mg/L |  |
| Inorganics | Metals | Metals by ICP | Manganese | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP | Nickel | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP | Potassium * | EPA 200.7, 4.4 (1994) | 0.1 mg/L |  |
| Inorganics | Metals | Metals by ICP | Sodium * | EPA 200.7, 4.4 (1994) | 0.1 mg/L |  |
| Inorganics | Metals | Metals by ICP | Titanium | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP | Vanadium | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP | Zinc | EPA 200.7, 4.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Antimony | EPA 200.8, 5.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Arsenic | EPA 200.8, 5.4 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Beryllium | EPA 200.8, 5.4 (1994) | 5 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Cadmium | EPA 200.8, 5.4 (1994) | 0.5 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Chromium | EPA 200.8, 5.4 (1994) | 5 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Cobalt | EPA 200.8, 5.4 (1994) | 50 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Copper | EPA 200.8, 5.4 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Lead | EPA 200.8, 5.4 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Manganese | EPA 200.8, 5.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Molybdenum | EPA 200.8, 5.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Nickel | EPA 200.8, 5.4 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Selenium | EPA 200.8, 5.4 (1994) | 1 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Silver | EPA 200.8, 5.4 (1994) | 1 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Strontium | EPA 200.8, 5.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Thallium | EPA 200.8, 5.4 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Tin | EPA 200.8, 5.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Vanadium | EPA 200.8, 5.4 (1994) | 10 ug/L |  |
| Inorganics | Metals | Metals by ICP/MS | Zinc | EPA 200.8, 5.4 (1994) | 10 ug/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Inorganics | Metals | Metals by GF/AA | Arsenic | EPA 200.9, Rev. 2.2 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by GF/AA | Cadmium | EPA 200.9, Rev. 2.2 (1994) | 0.5 ug/L |  |
| Inorganics | Metals | Metals by GF/AA | Copper | EPA 200.9, Rev. 2.2 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by GF/AA | Lead | EPA 200.9, Rev. 2.2 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by GF/AA | Nickel | EPA 200.9, Rev. 2.2 (1994) | 2 ug/L |  |
| Inorganics | Metals | Metals by GF/AA | Selenium | EPA 200.9, Rev. 2.2 (1994) | 1 ug/L |  |
| Inorganics | Metals | Metals by GF/AA | Silver | EPA 200.9, Rev. 2.2 (1994) | 1 ug/L |  |
| Inorganics | Metals | Mercury | Mercury | EPA 245.1 Rev. 3.0 (1994) | 0.2 ug/L |  |
| #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |
| Inorganics | Metals | Mercury | Mercury | EPA 245.6 Rev. 2.3 (1991) |  | 0.002 mg/Kg |
| Inorganics | Metals | Mercury | Low-Level Mercury | EPA 1631 E | 0.5 ng/L |  |
| Inorganics | Micro | Alk to pH 4.5, total as CaCO3 | Alkalinity 4.5 | SM 2320 B-1997 | 1 mg/L |  |
| Inorganics | Micro | Alk to pH 8.3, total as CaCO3 | Alkalinity 8.3 | SM 2320 B-1997 | 1 mg/L |  |
| Inorganics | Micro | BOD5 | BOD5 | SM 5210 B- 2001 | 2.0 mg/L |  |
| Inorganics | Micro | CBOD5 | CBOD5 | SM 5210 B- 2001 | 2.0 mg/L |  |
| Inorganics | Micro | Coliform, E.coli | Ecoli - MPN Analysis by Quantitray | SM 9222 D -1998 | 1 cfu/100 ml |  |
| Inorganics | Micro | Coliform, Fecal - MF | Coliform, Fecal - MF | SM 9222 D -1997 | 1 cfu/100 ml |  |
| Inorganics | Micro | Coliform, Total - MF | Coliform, Total - MF | SM 9222 B -1997 | 1 cfu/100 ml |  |
| Inorganics | Micro | Dissolved Organic Carbon | Dissolved Organic Carbon | SM 5310 B-2014 | 2 mg/L |  |
| Inorganics | Micro | Hardness | Hardness | SM 2340 C-2011 | 1 mg/L |  |
| Inorganics | Micro | Specific Conductance | Specific Conductance | SM 2510 B-1997 | 14.9 umhos/cm |  |
| Inorganics | Micro | Total Organic Carbon | Total Organic Carbon | SM 5310 B-2014 | 2 mg/L |  |
| Inorganics | Micro | Turbidity | Turbidity | SM 2130 B 2001 | 1 NTU |  |
| Inorganics | Nutrients | Ammonia | Ammonia | EPA 350.1, Rev. 2.0 (1993) | 0.1 mg/L |  |
| Inorganics | Nutrients | Nitrate + Nitrite | Nitrate + Nitrite | EPA 353.2, Rev. 2.0, 1993 | 0.1 mg/L |  |
| Inorganics | Nutrients | Nitrite | Nitrite | EPA 353.2, Rev. 2.0 (1993) | 0.02 mg/L |  |
| Inorganics | Nutrients | Orthophosphate | Orthophosphate | EPA 365.1, Rev. 2.0 (1993) | 0.05 mg/L |  |
| Inorganics | Nutrients | Total Kjeldahl Nitrogen | Total Kjeldahl Nitrogen | EPA 351.2, Rev. 20. (1993) | 0.6 mg/L |  |
| Inorganics | Nutrients | Total Phosphorus | Total Phosphorus | Lachat Quikchem 10-115-01-1-F | 0.05 mg/L |  |
| Inorganics | Wet Chem | Chemical Oxygen Demand | Chemical Oxygen Demand | SM 5220 C - 1997 | 20 mg/L |  |
| Inorganics | Wet Chem | Chlorophyll a | Chlorophyll a | EPA 445.0, Rev. 1.2, 1997 | 1 µg/L |  |
| Inorganics | Wet Chem | Color, ADMI | Natural ADMI | SM 2120 E-2011 | 10 c.u. |  |
| Inorganics | Wet Chem | Color, True | PTCO | SM 2120 B-2011 | 5 c.u. |  |
| Inorganics | Wet Chem | Cyanide | Cyanide | Kelada-01 Rev. 1.2 (2001) | 0.005 mg/L |  |
| Inorganics | Wet Chem | Hexavalent Chromium | Hexavalent Chromium | EPA 218.6, Rev. (1994) | 0.2 ug/L |  |
| Inorganics | Wet Chem | Ion Chromatography | Bromide | EPA 300.0, Rev.2.1 (1993) | 0.4 mg/l |  |
| Inorganics | Wet Chem | Ion Chromatography | Chloride | EPA 300.0, Rev.2.1 (1993) | 1 mg/l |  |
| Inorganics | Wet Chem | Ion Chromatography | Fluoride | EPA 300.0, Rev.2.1 (1993) | 0.4 mg/l |  |
| Inorganics | Wet Chem | Ion Chromatography | Sulfate | EPA 300.0, Rev.2.1 (1993) | 2 mg/l |  |
| Inorganics | Wet Chem | MBAS | MBAS | SM5540 C-2000 | 0.1 mg/L |  |
| Inorganics | Wet Chem | Microcystins | Microcystins | EPA 546, Rev. (2016) | 0.4 ug/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Inorganics | Wet Chem | Oil & Grease | Oil & Grease | EPA 1664 B | 10 mg/L |  |
| Inorganics | Wet Chem | Oil & Grease | Oil & Grease | SW-846-9071 B |  |  |
| Inorganics | Wet Chem | Phenol, Inorganic | Phenol, Inorganic | EPA 420.4, Rev. (1994) | 10 ug/L |  |
| Inorganics | Wet Chem | Residue, Suspended Fixed | Residue, Suspended Fixed | SM 2540 E-1997 | 2.5 mg/L |  |
| Inorganics | Wet Chem | Residue, Suspended Volatile | Residue, Suspended Volatile | SM 2540 E-1997 | 2.5 mg/L |  |
| Inorganics | Wet Chem | Residue, Total | Residue, Total | SM 2540 B-1997 | 12 mg/L |  |
| Inorganics | Wet Chem | Residue, Total Dissolved | Residue, Total Dissolved | SM 2540 C 1997 | 12 mg/L |  |
| Inorganics | Wet Chem | Residue, Total Fixed | Residue, Total Fixed | SM 2540 E-1997 | 12 mg/L |  |
| Inorganics | Wet Chem | Residue, Total Suspended | Residue, Total Suspended | SM 2540 D-1997 | 2.5 mg/L |  |
| Inorganics | Wet Chem | Residue, Total Volatile | Residue, Total Volatile | SM 2540 E-1997 | 12 mg/L |  |
| Inorganics | Wet Chem | Silica | Silica | SM 4500-SiO2 C-2011 | 2 mg/L |  |
| Inorganics | Wet Chem | Sulfide | Sulfide | SM 4500-S2 D&F-2000 | 0.1 mg/L |  |
| Inorganics | Wet Chem | Tannin & Lignin | Tannin & Lignin | 0 | 0.2 mg/L |  |
| Organics | Pesticides | Organochlorine Pesticides | A-BHC | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | A-Chlordane | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Aldrin | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | B-BHC | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | D-BHC | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Dieldrin | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Endosulfan I | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Endosulfan II | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Endosulfan Sulfate | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Endrin | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Endrin Aldehyde | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Endrin Ketone | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | G-BHC | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | G-Chlordane | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Heptachlor | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Heptachlor Expoxide | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | Methoxychlor | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | p,p-DDD | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | p,p-DDE | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organochlorine Pesticides | p,p-DDT | EPA 608.3 | 0.05 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Azinphos Methyl | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Chlorpyrifos | EPA 614 | 0.5 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Demeton | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Diazinon | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Disulfoton | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Ethion | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Ethyl Parathion | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organophosphorus Pesticides | Malathion | EPA 614 | 2 Units: |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | Pesticides | Organophosphorus Pesticides | Methyl Parathion | EPA 614 | 2 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Ametryn | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Atraton | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Atrazine | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Prometryn | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Propazine | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Secbumeton | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Simetryn | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Terbuthylazine | EPA 619 | 10 Units: |  |
| Organics | Pesticides | Organonitrogen Pesticides | Terbutryn | EPA 619 | 10 Units: |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | 11Cl-PF3OUdS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | 4:2 FTS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | 6:2 FTS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | 8:2 FTS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | 9Cl-PF3ONS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | HFPO-DA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | NaDONA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | N-EtFOSAA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | N-MeFOSAA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFBA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFBS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFDA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFDoA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFDoS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFDS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFHpA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFHpS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFHxA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFHxS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFNA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFNS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFOA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFOS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFPeA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFPeS | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFTeDA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFTrDA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFUnA | DWRPFAS | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | Byproduct 1 | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | ByProduct 2 | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | Byproduct 4 | DWRPFAS2022 | 2 ng/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | Byproduct 5 | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | Byproduct 6 | DWRPFAS2022 | 2 ng/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | Hydro-EVE | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | NVHOS | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PEPA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFECA G | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFECA-B | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFEESA/PES | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFMOAA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFO2HxA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFO3OA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFO4DA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PFO5DA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | PMPA | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Per- and Polyfluoroalkyl Substances | R-EVE | DWRPFAS2022 | 2 ng/L |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | 4-Chloroaniline | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Acenaphthene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Acenaphthylene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Aniline | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Anthracene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzo(a)anthracene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzo(a)pyrene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzo(b)fluroanthene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzo(g,h,i)perylene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzo(k)fluoranthene | EPA 625.1 | 30 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzoic acid | EPA 625.1 | 50 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Benzyl alcohol | EPA 625.1 | 30 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Bis(2-chlorethyl)ether | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Bis(2-chloroethoxy)methane | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Bis(2-chloroisopropyl)ether | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Bis(2-ethylhexyl)phthalate | EPA 625.1 | 30 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Bromophenyl phenyl ether, 4- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Butylbenzyl phthalate | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Chloro-3-Methylphenol, 4- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Chloronaphthalene, 2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Chlorophenol, 2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Chlorophenyl phenyl ether, 4- | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Chrysene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dibenzo(a,h)anthracene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dibenzofuran | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dichlorobenzene, 1,2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dichlorobenzene, 1,3 | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dichlorobenzene, 1,4- | EPA 625.1 | 10 ug/mL |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dichlorobenzidine, 3,3'- | EPA 625.1 | 30 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dichlorophenol, 2,4- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Diethyl phthalate | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dimethyl phthalate | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dimethylphenol, 2,4- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Di-n-butyl phthalate | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dinitro-2-Methylphenol, 4,6- | EPA 625.1 | 50 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dinitrophenol, 2,4- | EPA 625.1 | 50 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dinitrotoluene, 2,4- | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Dinitrotoluene, 2,6- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Di-n-octyl phthalate | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Fluoranthene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Fluorene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Hexachlorobenzene (HCB) | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Hexachlorobutadiene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Hexachlorocyclopentadiene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Hexachloroethane | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Indeno(1,2,3-cd)-Pyrene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Isophorone | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Methylnaphthalene, 2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Methylphenol, 2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Methylphenol, 4- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Naphthalene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Nitroaniline, 2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Nitroaniline, 4- | EPA 625.1 | 50 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Nitrobenzene | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Nitrophenol, 2- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Nitrophenol, 4- | EPA 625.1 | 50 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | N-Nitrosodiphenylamine | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | N-Nitrosodipropylamine | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Pentachlorophenol | EPA 625.1 | 30 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Phenanthrene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Phenol | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Pyrene | EPA 625.1 | 20 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Trichlorobenzene, 1,2,4- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Trichlorophenol, 2,4,5- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Trichlorophenol, 2,4,6- | EPA 625.1 | 10 ug/mL |  |
| Organics | SVOA | Semi-Volatile Organic Analytes | Nitroaniline 3- | EPA 625.1 | 50 ug/mL |  |
| Organics | SVOA | Deisel Range Total Petroleum Hydrocarbons | Deisel Range Total Petroleum Hydrocarbons | SW-846 8015 C | 0.5 mg/L |  |
| Organics | VOA | Volatile Organic Analytes | Benzene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Bromobenzene | EPA 624.1 | 1 ug/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | VOA | Volatile Organic Analytes | Bromochloromethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Bromodichloromethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Bromoform | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Bromomethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Butylbenzene, n- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Butylbenzene, sec- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Butylbenzene, t- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Carbon Tetrachloride | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Chlorobenzene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Chloroethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Chloroform | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Chloromethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Chlorotoluene, 2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Chlorotoluene, 4- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dibromo-3-Chloropropane, 1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dibromochloromethane | EPA 624.1 | 1 ug/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | VOA | Volatile Organic Analytes | Dibromoethane, 1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dibromomethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichlorobenzene, (1,2)- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichlorobenzene, (1,3)- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichlorobenzene, (1,4)- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloroethane, 1,1- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloroethane, 1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloroethene, 1,1- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloroethene, cis-1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloroethene, trans-1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichlorofluoromethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloropropane, 1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloropropane, 1,3- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloropropane, 2,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloropropene, cis-1,3- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichloropropene, trans-1,3- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Dichlropropene, 1,1- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Dioxane, 1,4- | Dioxane, 1,4- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Ethylbenzene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Hexachlorobutadiene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Isopropylbenzene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Isopropyltoluene, p- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Methylene chloride | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Methyl-t-butyl ether | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Naphthalene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Propylbenzene, n- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Styrene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Tetrachloroethane, 1,1,1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Tetrachloroethene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Tetrachlorothane, 1,1,2,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Toluene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichlorobenzene, 1,2,3- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichlorobenzene, 1,2,4- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichloroethane, 1,1,1- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichloroethane, 1,1,2- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichloroethene | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichlorofluoromethane | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trichloropropane, 1,2,3- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trimethylbenzene, 1,2,4- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Trimethylbenzene, 1,3,5- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Vinyl Chloride | EPA 624.1 | 1 ug/L |  |

| Dept | Unit | Parameter | Analyte | Analysis Method | PQLs |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Aqueous | Tissue |
| Organics | VOA | Volatile Organic Analytes | Xylene, m,p- | EPA 624.1 | 2 ug/L |  |
| Organics | VOA | Volatile Organic Analytes | Xylene, o- | EPA 624.1 | 1 ug/L |  |
| Organics | VOA | Gasoline Range Total Petroleum Hydrocarbons | Gasoline Range Total Petroleum Hydrocarbons | SW-846 8015 C | 0.2 mg/L |  |