Tiered Proposal for Post-Construction Nutrient Stormwater Control

Project-Level Objectives for Stormwater Rule

- Provide conventional pollutant treatment of water quality volume

- No increase in nutrient loading from average undeveloped condition

- Protect physical integrity and biota of receiving streams

  - Reduce post-development runoff volume increase by requiring ET/I for some fraction of treatment volume

  - Avoid increases to duration of channel-forming discharge rate

Some Observations

- Disconnecting BUA provides greater opportunity for runoff reduction

- “Car habitat” BUA – parking, roads, driveways - yields greater unit nutrient loads than rooftop

- BUA impact on streams at watershed-scale:

  - Stream communities experience 50% reduction in diversity with 10% impervious cover in watershed

  - At 25% impervious cover streams become “non-supporting”

Proposal

- Stormwater management tiers based on % BUA, extent of runoff concentration (curb and gutter vs open), and cover type nutrient yield differences

- Strategies:

  - Starting at lower %BUA than most state stormwater, require some level of control/treatment. Increase treatment as increase BUA intensity.

  - In open drainage settings, take advantage of IC disconnection and flow dispersal opportunities in lieu of more rigorous, conventional treatment requirements

  - Add some level of infiltration/groundwater recharge to typical treatment (SWMM modeling results)

  - Also evaluate some form of control for channel-forming discharge events (1-2 yr) via SWMM modeling analysis

|  | Open Drainage (No curb & gutter) | Curb & gutter |
| --- | --- | --- |
| < 6% BUA | Exempt | Not allowed |
| 6% < BUA < 12% | - Vegetated conveyances only - Disconnect IC from conveyances - Disperse IC-concentrated flows - Treat “car habitat” IC w/runoff-reducing 2⁰ SCM or 1⁰ SCM | Treat all IC w/1⁰ SCM per SWMM model rec’s |
| 12% < BUA < 24% | - Vegetated conveyances only - Treat all IC w/runoff-reducing 1⁰ SCM or w/1⁰ SCM per SWMM model rec’s | - Treat all IC w/1⁰ SCM per SWMM model rec’s - Rate control for stream protection per SWMM model rec’s |
| BUA > 24% | N/A (doesn’t happen) | Same as above * |

* For infill development where surrounding development > 60% BUA, option to utilize offsite offsets for both treatment and volume control

Runoff-reducing 2⁰ SCMs: DIS; LS-FS

Runoff-reducing 1⁰ SCMs: Bioretention; Infiltration; Permeable Pavement - Infiltrating; Silva Cell; Rainwater Harvesting

How is proposal same as current requirements?

- Curb and gutter is supposed to have 1⁰ SCM treatment regardless of BUA?

- Open drainage requirements are similar to current 2H Low Density (except for SCMs)

- Uses 6% BUA-increment thresholds used in current 2H

How is proposal different?

- Treatment required starting at 6% BUA; currently only WS-II CA this protective

- Tiered, increasingly stringent treatment as %BUA increases

  - Lesser requirements for open drainage development vs C&G at given BUA

  - Volume-reducing 2⁰SCM sufficient at low-BUA (open drainage development)

  - Car-BUA treatment required before other BUA (open drainage development)

  - Add rate control requirement at higher density

- Eased onsite requirements for high-density to accommodate downtown-type infill