|  |  | No curb & gutter within proposed development, including new proposed roads | Curb & gutter included in proposed development |
| --- | --- | --- | --- |
| BUA < 6% |  | - Vegetated conveyances only - Disconnect impervious cover from conveyances - Disperse IC-concentrated flows | Not applicable |
| 6% ≤ BUA < 12% |  | - Vegetated conveyances only - Disconnect impervious cover from conveyances - Disperse IC-concentrated flows - Treat transportation impervious cover w/ primary SCM or runoff-reducing secondary SCM | - Treat site runoff from 1" storm w/ primary SCM, including volume reduction requirement (see below), calculated w/ curve number method OR - Treat site runoff from 90th percentile storm w/ primary SCM |
| 12% ≤ BUA | Stream Protection Criteria | Potential Options for Initial Discussion - Control the 2yr/24hr post-development peak flow rate to 50% of the 2yr/24hr pre-development level OR - Control the 2yr/24hr post-development peak flow rate to the 1yr/24hr predevelopment level OR - Detain the volume difference between the post-development 1yr/24hr storm and the pre-development 1yr/24hr storm, releasing the volume over 24 hours |  |
|  | Water Quality Treatment Criteria | - Treat site runoff from 1" storm w/ primary SCM, including volume reduction requirement (see below), calculated w/ curve number method OR - Treat site runoff from 90th percentile storm w/ primary SCM |  |

Transportation Impervious Cover: Uncovered, paved or hardened surfaces used by vehicles, including parking areas, driveways, and roads

Primary SCMs: Bioretention, Infiltration, SW Wetland, Permeable Pavement, Wet Pond, Sand Filter, Rainwater Harvesting, StormFilter, Silva Cell, others specified in NC Stormwater Design Manual

Runoff-reducing Secondary SCMs: DIS; LS-FS; Treatment Swale;

Volume Reduction Requirement (SCS method)

Portion of runoff from 1" storm that is required to be achieved via evapotranspiration, infiltration, or slow filtered discharge

| Hydrologic Soil Group | Volume Reduction Requirement |
| --- | --- |
| A | 0.38 |
| B | 0.26 |
| C | 0.13 |
| D | 0.07 |

Stream Protection Criteria

Options:

- Control the 2-year, 24-hour post-development peak flow rate to 50 percent of the 2-year, 24-hour pre-development level

- Control the 2-year, 24-hour post-development peak flow rate to the 1-year, 24-hour predevelopment level

- 24-hour extended detention of post-developed 1-year, 24-hour storm event

Exemptions:

- The entire channel protection volume is recharged to groundwater

- Sites less than or equal to one acre of impervious cover

- Compliance with the stream protection criteria above can be demonstrated to result in no benefit to current and future downstream development